# Lab 6: Setting up OM2M and Communication with ESP32

Spring-2025
February 20, 2025

## Student Details

Name: Samyak Jain, Rishabh Goyal
Room No.: N125
Table No.: 42

## AIM

To set up OM2M, communicate with an ESP32, and implement data exchange between the ESP32 and an OM2M server for remote monitoring of CPU and RAM usage.

## Software Tools Required

- Postman REST-API Client
- OM2M Server
- Java JDK 1.8
- Python
- Jupyter Notebook

## Hardware Tools Required

- 1 ESP32
- 1 LED
- 2 Laptops
- Stable WiFi connection

## Experiment 1: Setting up Postman, OM2M Server, and Jupyter Notebook

### Procedure

1. Postman Installation:
   - Download and install Postman.
   - Import the OM2M REST API collection for testing.

2. OM2M Server Setup:
   - Download and extract OM2M.
   - Modify config.ini to set the port to 5089.
   - Start the OM2M server and log in via http://127.0.0.1:5089/webpage.

3. Python and Jupyter Notebook Setup:
   - Install Python and Jupyter Notebook.
   - Set up a virtual environment and install required dependencies.

Creating the OM2M Resource tree using postman

- Go to the Postman then click Workspaces then press Import then Upload Files and select the JSON files named as ‘OM2M REST APIs.postman collection.json’

- Click on Import. We can see the request collections below the OM2M Rest APIs

- Before using the collection make sure the OM2M server is running and available at http://127.0.0.1: 5089/webpage. The request can be sent by pressing the “Send” button in postman, after selecting any request.

- Now send each of the following commands to construct the resource tree

- Creating an Application Entity: Select Post Create Application Entity Press Send

- Creating a container for device called Node-1: Select Post Create Node-1 container Press Send

- Creating a Descriptor container: Select Post create descriptor cnt Press Send

- Creating a Data container: Select Post Create Data Container Press Send

- Creating descriptor content instance: Select Post create descriptor cin Press Send

- Creating data content instance 1 & 2: Select Post Create Content instance 1/2 Press Send

- Retrieving the latest content instance: Select GET latest Content instance Press Send

- Retrieving the oldest content instance: Select GET oldest Content instance Press Send

- Retrieving all content instances: Select GET all data of a container Press Send

- Updating number of attributes inside data container: Select PUT update cnt attrs Send.

### Observations

- The OM2M resource tree was successfully created.
- Postman successfully sent API requests to OM2M.
- Jupyter Notebook was successfully configured.

- resource tree was formed

### Results/Conclusions

OM2M and Postman were set up successfully, allowing device resource management through API requests.

## Experiment 2: Publishing CPU and RAM Usage Data to OM2M

### Procedure

1. Using Jupyter Notebook as a Mock Device:
   - Launch Jupyter Notebook and run "Mock Device CPU Usage.ipynb".
   - Publish CPU and RAM usage data to OM2M.

2. OM2M Resource Tree Updates:
   - Send API requests to create an application entity and data container.
   - Retrieve and visualize stored data.

### Observations

- CPU and RAM usage data was successfully published to OM2M.
- Data updates appeared in the OM2M resource tree.

### Results/Conclusions

The system successfully simulated data publishing from a device to OM2M.

## Experiment 3: Implementing a Remote CPU and RAM Usage Monitor

### Procedure

1. ESP32 Data Retrieval from OM2M:
   - ESP32 fetches the latest CPU and RAM usage data via an HTTP GET request.
   - Data is printed to the serial monitor.

2. LED Blinking Based on CPU/RAM Threshold:
   - If CPU or RAM usage exceeds a defined threshold, the LED blinks rapidly.
   - The LED stops blinking once values return to a safe range.

### Observations

- ESP32 successfully retrieved data from OM2M.
- LED responded correctly to threshold exceedances.

### Results/Conclusions

The ESP32 successfully monitored and reacted to CPU and RAM usage changes.